# Jetson Nano LED Control using Sockets and Tkinter GUI

# Overview:

This project allows users to remotely control an LED connected to a Jetson Nano using a client-server architecture. The Jetson Nano acts as a server, receiving commands from a client device that has a graphical user interface (GUI) built with Tkinter. The communication between the server and client is handled using Python sockets.

# Features :

- Remote LED control over a local network

- Simple client-side GUI with a toggle button

- Uses Python sockets for communication

- Works with Jetson Nano's GPIO pins

## Components Required

- Jetson Nano

- LED

- 220Ω resistor

- Breadboard and jumper wires

- Tkinter (for GUI on the client side)

- Jetson.GPIO (for controlling the GPIO on the Jetson Nano)

## System Architecture

- The Jetson Nano acts as a server, listening for incoming commands ("on" or "off").

- The client device (another computer or Raspberry Pi) runs a GUI application to send LED control commands.

- Communication happens over a local network using Python sockets.

Circuit

- Connect the long leg (anode) of the LED to a GPIO pin (e.g., GPIO 12) on the Jetson Nano through a 330Ω resistor.

- Connect the short leg (cathode) to the ground (GND) pin of the Jetson Nano.

## Packages need to be installed:

Install the following packages on Jetson nano

- Pip newer version

- Jetson.GPIO

install the following package on client system

- sudo apt install python3-tk

Connect the server and client in same local network

## Server-side Code (Jetson Nano)

import Jetson.GPIO as GPIO

import socket

import time

# Initialize GPIO

GPIO.setmode(GPIO.BOARD)

GPIO.setup(12, GPIO.OUT)

# Create a socket object

server_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

​

# Get the IP address of the Jetson Nano

host = '0.0.0.0'  # Listen on all available network interfaces

port = 12345  # Port to bind to

​

# Bind the socket to the address

server_socket.bind((host, port))

​

# Listen for incoming connections

server_socket.listen(5)

​

print(f"Server is running on {host}:{port}")

​

while True:

# Accept a new connection

client_socket, client_address = server_socket.accept()

print(f"Connection from {client_address}")

​

try:

# Receive the data sent by the client

data = client_socket.recv(1024).decode()

​

if data == 'on':

GPIO.output(12, GPIO.HIGH)  # Turn LED on

print("LED ON")

elif data == 'off':

GPIO.output(12, GPIO.LOW)  # Turn LED off

print("LED OFF")

else:

print(f"Invalid command: {data}")

​

except Exception as e:

print(f"Error: {e}")

finally:

client_socket.close()

​

# Clean up GPIO on exit

GPIO.cleanup()

## Client-side Code (GUI with Tkinter):

import socket

import tkinter as tk

​

# Function to send command to the server

def send_command(command):

try:

# IP address of the Jetson Nano (Server)

host = '192.168.1.4'  # Replace with your Jetson Nano's IP address

port = 12345  # Port used by the server

​

# Create a socket object

client_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

​

# Connect to the server

client_socket.connect((host, port))

​

# Send the command (either 'on' or 'off') to control the LED

client_socket.send(command.encode())

client_socket.close()

except Exception as e:

print(f"Error: {e}")

​

# Function to toggle LED state

def toggle_led():

if toggle_button["text"] == "Turn LED ON":

send_command("on")  # Send 'on' command

toggle_button.config(text="Turn LED OFF")  # Update button text

else:

send_command("off")  # Send 'off' command

toggle_button.config(text="Turn LED ON")  # Update button text

​

# Create the GUI window

root = tk.Tk()

root.title("LED Control")

​

# Add a toggle button to control the LED

toggle_button = tk.Button(root, text="Turn LED ON", command=toggle_led, height=2, width=20)

toggle_button.pack(pady=20)

​

# Run the GUI loop

root.mainloop()

​

### How to Run the Project:

  - Run the Server on Jetson Nano

  - Run the Client on Any Device

  - Use the GUI

    - Click the “LED ON” button to turn the LED on

    - Click the “LED OFF” button to turn the LED off

.